ESP32 Drone Project

Project Documentation / Hardware Datasheet / Build & Operation Guide

1. Project Overview

I originally started this project with a small wooden drone kit that I had purchased several years ago. The drone crashed during its first test, and the project was left unfinished for a long time.

Later, I decided to take the drone apart and investigate how it worked instead of simply replacing the entire system. I studied the original flight-control PCB, identified its major components, traced important connections, and measured the power rails and signal paths with a multimeter.

After successfully repairing the original PCB, I decided to take the project further. Instead of only repairing the original drone, I began designing my own drone PCB based on an ESP32.

This project has therefore developed from a simple drone repair into a complete embedded-electronics project involving PCB design, microcontrollers, sensors, power management, motor control, firmware, debugging, and manufacturing.

The final goal is to build a drone system that I understand from the individual electronic components all the way to the complete flight-control system.

2. Project Goals

My main goals for this project are:

- Design my own drone flight-control PCB.

- Use an ESP32 as the main microcontroller.

- Integrate motion and altitude sensors.

- Control four motors.

- Build a proper onboard power system.

- Provide interfaces for additional sensors.

- Provide test points for debugging.

- Learn practical PCB design and manufacturing.

- Develop my own drone-control firmware.

- Learn how flight-control hardware and software interact.

- Create a hardware platform that can be improved through future PCB revisions.

The purpose is not simply to make a drone fly. I want to understand the complete system, from the schematic and PCB to the firmware and physical testing.

3. Project History

3.1 Original Drone

The project originally started with a small wooden-shell drone.

During its first test, the drone crashed. I kept the hardware and eventually returned to the project several years later.

Instead of throwing away the damaged drone, I decided to investigate the original electronics.

3.2 Reverse Engineering

I examined the original flight-control PCB and identified several important areas:

- Main MCU

- IMU

- Barometric pressure sensor

- Motor-control circuitry

- LEDs

- Battery input

- Communication interfaces

- Power regulation

I used a multimeter to check continuity, voltage levels, and possible short circuits.

I also investigated the PCB traces to understand how the components were connected.

The original board contained several components whose exact functions were not immediately obvious. I therefore used the PCB markings, measurements, component markings, and circuit behavior to identify their likely functions.

3.3 Original PCB Repair

After investigating the board and correcting the problems, I successfully powered the original PCB.

The board operated normally, and the status LED worked correctly.

I also measured the motor output terminals. Without the motors connected, the outputs showed approximately battery-level voltage.

This confirmed that the original control electronics were functioning.

The repair was an important step because it allowed me to understand how a real drone PCB was designed before starting my own design.

4. From Repair to New PCB

After repairing the original board, I decided that I wanted to build a new system rather than simply continue using the original PCB.

I started learning KiCad and created a new schematic and PCB layout.

The new design is based around an ESP32 and is intended to be more flexible than the original toy-drone controller.

The new PCB is designed to support:

- Microcontroller control

- Motion sensing

- Altitude sensing

- Distance sensing

- Motor control

- Battery power

- Voltage regulation

- Debugging

- Wireless communication

- Future telemetry

- Future autonomous functions

5. System Architecture

The overall system can be divided into several sections:

LiPo Battery

|

v

+----------------+

| Power System   |

| Regulation     |

+----------------+

|         |

|         v

|      Sensors

|     +-------+

|     | IMU   |

|     | BMP280|

|     | ToF   |

|     +-------+

|

v

+-------------+

|    ESP32    |

| Main MCU    |

+-------------+

|   |   |

-------    |    -------

|           |            |

v           v            v

Motor 1-4    UART/I2C    Status LEDs

|

v

Motor Drivers

|

v

Motors

The ESP32 acts as the central controller.

Sensors provide information to the ESP32, the ESP32 processes the information, and the motor-control circuitry converts the control signals into electrical power for the motors.

6. Main Controller

ESP32

The ESP32 is the main microcontroller of this project.

It is responsible for:

- Running the firmware

- Reading sensors

- Processing sensor data

- Controlling motors

- Managing communication

- Monitoring system status

- Handling future wireless functions

I chose the ESP32 because it provides significantly more processing capability and connectivity than the original toy-drone controller.

It also provides:

- GPIO

- I2C

- UART

- SPI

- PWM

- Wi-Fi

- Bluetooth on supported ESP32 variants

- Hardware timers

- Interrupt support

These features provide enough flexibility for future versions of the drone.

7. Component Selection

7.1 ESP32

The ESP32 is selected as the main controller.

When selecting the exact ESP32 module or development board, I consider:

- Flash capacity

- PSRAM requirements

- GPIO availability

- Wireless functions

- Module size

- Power consumption

- Programming interface

- Availability

- PCB footprint

For the final custom PCB, the module footprint must match the exact ESP32 module being used.

8. IMU

The IMU is used to measure the movement and orientation of the drone.

An IMU typically contains:

- Accelerometer

- Gyroscope

The accelerometer measures linear acceleration.

The gyroscope measures angular velocity.

These measurements can be processed by the ESP32 to estimate the drone's attitude and movement.

A BMI088-class sensor is suitable for this type of application because it provides a dedicated accelerometer and gyroscope and is designed for motion-sensing applications.

The IMU should be placed close to the mechanical center of the flight controller whenever practical.

I avoid placing it directly next to:

- High-current motor traces

- Switching power components

- Large vibrating mechanical structures

- Strong electromagnetic noise sources

9. Barometric Pressure Sensor

The project includes a BMP280-class barometric pressure sensor.

The sensor measures atmospheric pressure.

Because atmospheric pressure changes with altitude, the ESP32 can use pressure measurements to estimate changes in altitude.

Possible applications include:

- Altitude monitoring

- Altitude-hold experiments

- Sensor fusion

- Flight-control development

The barometer should not be placed where motor airflow or strong turbulence directly affects the pressure measurement.

10. Distance Sensor

The PCB design also provides support for a VL53-series time-of-flight distance sensor.

A ToF sensor measures distance using light.

Possible applications include:

- Low-altitude distance measurement

- Experimental altitude control

- Obstacle detection

- Landing assistance

- Future autonomous functions

The sensor should have a clear optical path.

Components, PCB structures, and mechanical parts should not block the sensor's field of view.

11. Motor System

The drone uses four motors in a quadcopter configuration.

The four channels are:

M1

M2

M3

M4

Each channel has its own motor-control path.

The motor-control system contains:

- ESP32 control signal

- MOSFET

- Gate-control components

- Motor power connection

- Ground return

- Supporting capacitors where required

The MOSFET acts as an electronic switch.

The ESP32 does not directly supply the motor current.

Instead:

ESP32

|

| Control signal

v

MOSFET

|

| Switches motor current

v

Motor

This separation is important because the ESP32 GPIO pins are intended for logic-level signals, not for directly driving motors.

12. Choosing the MOSFET

The MOSFET should be selected based on the actual motor current.

Important specifications include:

- Drain-source voltage rating

- Continuous drain current

- Pulse current capability

- RDS(on)

- Gate threshold

- Gate-drive voltage

- Package size

- Thermal characteristics

A low RDS(on) device is desirable because it reduces conduction losses.

However, the specified gate threshold voltage should not be confused with the voltage at which the MOSFET is fully enhanced.

For an ESP32-controlled design, I select a MOSFET that is properly specified for the actual GPIO drive voltage.

13. Power System

The drone is powered by a rechargeable lithium battery.

The battery provides the main input power to the PCB.

The PCB then uses voltage-regulation circuitry to generate the appropriate voltage rails for the ESP32 and other electronics.

The power system contains:

- Battery input

- Power distribution

- Voltage regulation

- Decoupling capacitors

- Filtering

- Test points

- Ground connections

The motor power path and logic power path should be considered separately.

Motors can cause voltage fluctuations and electrical noise, so sensitive sensors and the ESP32 should receive appropriately regulated and filtered power.

14. Capacitors

Capacitors are used throughout the PCB for:

- Power decoupling

- Voltage stabilization

- Noise reduction

- Transient current support

- Filtering

Small ceramic capacitors should generally be placed close to IC power pins.

For example:

Power Rail

|

+---- IC VCC

|

=== Capacitor

|

GND

The physical distance between the capacitor and the IC power pin matters because PCB trace inductance affects high-frequency decoupling performance.

15. Resistors

Resistors are used for several purposes:

- Pull-up resistors

- Pull-down resistors

- Signal conditioning

- MOSFET gate control

- LED current limiting

- Sensor configuration

- Protection and measurement circuits

The exact resistance depends on the circuit.

For example, an LED should not normally be connected directly to a GPIO pin without appropriate current limiting.

16. LEDs

LEDs are included for status indication and debugging.

Depending on the firmware, LEDs can indicate:

- Power

- Startup

- Sensor status

- Communication status

- Errors

- Debugging information

A current-limiting resistor is used with each LED.

A typical connection is:

ESP32 GPIO

|

Resistor

|

LED

|

GND

The resistor value is selected according to the LED forward voltage and desired current.

17. Test Points

I included test points in the PCB design so that important electrical signals can be measured directly.

Test points are useful for:

- Checking power rails

- Checking battery voltage

- Checking signal levels

- Debugging communication

- Investigating unexpected behavior

- Verifying the PCB after manufacturing

For example, I can provide test points for:

BATT+

GND

3.3V

I2C

UART

Motor control signals

The exact test points depend on the PCB revision.

18. Communication Interfaces

I2C

I2C can be used for:

- IMU

- BMP280

- VL53-series sensor

- Other compatible sensors

Typical I2C connections are:

ESP32

|

+---- SDA

|

+---- SCL

|

+---- GND

|

+---- VCC

I2C normally requires appropriate pull-up resistors on SDA and SCL.

UART

UART can be used for:

- Debugging

- Serial communication

- Configuration

- Telemetry

- External modules

During development, UART is particularly useful because it allows the ESP32 to output sensor values and system status.

19. PCB Construction Method

I design the PCB in KiCad.

The general workflow is:

Idea

↓

Circuit design

↓

Schematic

↓

Component selection

↓

Footprint assignment

↓

PCB layout

↓

Routing

↓

DRC

↓

Gerber

↓

Drill files

↓

ZIP

↓

PCB manufacturing

↓

Assembly

↓

Testing

20. Creating the Schematic

I first create the schematic before routing the PCB.

The schematic contains:

- ESP32

- Sensors

- MOSFETs

- Resistors

- Capacitors

- Connectors

- Power regulators

- LEDs

- Test points

I assign meaningful net names such as:

GND

BATT+

3V3

SDA

SCL

TX

RX

M1

M2

M3

M4

Clear net names make debugging significantly easier.

21. Footprint Selection

Each schematic component needs an appropriate PCB footprint.

When selecting a footprint, I consider:

- Physical dimensions

- Pin pitch

- Package type

- Manufacturer datasheet

- Assembly method

- Availability

- PCB manufacturing capabilities

The footprint must match the actual physical component.

A visually similar footprint is not necessarily electrically or mechanically compatible.

For critical components such as the ESP32 module, sensors, connectors, and MOSFETs, I verify the footprint against the manufacturer's datasheet.

22. PCB Placement

I place important components before routing.

A typical priority is:

- ESP32

- IMU

- Power circuitry

- Motor-control circuitry

- Connectors

- Other sensors

- Capacitors

- Resistors

- LEDs

- Test points

The exact arrangement depends on the mechanical design.

The IMU should be positioned appropriately for flight dynamics.

The ESP32 antenna area should have sufficient clearance.

High-current motor traces should not unnecessarily pass through sensitive sensor areas.

23. PCB Routing

I route the most important connections first.

Typical priority:

- Battery power

- Motor power

- Ground

- Regulated power rails

- Sensor buses

- MCU signals

- LEDs

- Debug signals

Power traces carrying higher currents should generally be wider than ordinary signal traces.

The exact width depends on:

- Current

- Copper thickness

- Trace length

- Temperature rise

- PCB manufacturer capabilities

24. Ground Plane

A ground plane is useful for providing a low-impedance return path and improving signal integrity.

For a compact PCB, I generally use a large GND copper area where practical.

However, the ground design still needs to account for high-current motor paths and sensitive analog or sensor signals.

The goal is not simply to connect everything to GND, but to provide sensible current-return paths.

25. Design Rule Check

Before manufacturing, I run KiCad's Design Rule Check.

I inspect:

- Clearance violations

- Unconnected items

- Track-width problems

- Hole-size problems

- Board-edge problems

- Footprint issues

- Silkscreen conflicts

I correct actual electrical and manufacturing problems before producing the final manufacturing files.

Some warnings, such as minor silkscreen overlaps, may not affect electrical operation, but I still inspect them individually rather than blindly ignoring all warnings.

26. Gerber Generation

After the PCB is complete, I generate Gerber files.

In KiCad:

PCB Editor

→ File

→ Fabrication Outputs

→ Gerbers

I select the required copper, solder mask, silkscreen, and board-outline layers.

I then generate the files.

27. Drill File Generation

I also generate the drill files.

In KiCad:

Gerber Generation

→ Generate Drill Files

For a normal PCB manufacturing workflow, I use:

- Excellon drill format

- PTH drill data

- NPTH drill data if required

I normally leave unusual alternative drill options at their default setting unless the manufacturer specifically requires another format.

28. Manufacturing ZIP

The final manufacturing package contains the Gerber and drill files.

The package should look approximately like:

ESP32_Drone_Gerber.zip

├── F_Cu.gbr

├── B_Cu.gbr

├── F_Mask.gbr

├── B_Mask.gbr

├── F_Silkscreen.gto

├── B_Silkscreen.gbo

├── Edge_Cuts.gm1

├── PTH.drl

└── NPTH.drl

The exact filenames depend on the KiCad version and project configuration.

I use an English filename for the ZIP package, such as:

ESP32_Drone_Gerber.zip

I then upload the ZIP to the PCB manufacturer.

29. PCB Assembly

After receiving the manufactured PCB, I first inspect it before soldering components.

I check:

- PCB dimensions

- Board outline

- Mounting holes

- Connector positions

- Silkscreen

- Copper layers

- Solder mask

- Plated holes

- Visible manufacturing defects

I compare the physical PCB against the KiCad design.

30. Component Assembly Method

I assemble the board systematically instead of installing every component at once.

A possible assembly order is:

Step 1 — Small Passive Components

Install:

- Resistors

- Small capacitors

- Other small passive components

Step 2 — Power Components

Install:

- Regulators

- MOSFETs

- Power-related components

Step 3 — Connectors

Install:

- Battery connector

- Motor connectors

- Programming connector

- Sensor connectors

Step 4 — Main Controller

Install the ESP32 module.

Step 5 — Sensors

Install:

- IMU

- BMP280

- VL53-series sensor

Step 6 — LEDs

Install the status LEDs and verify their polarity.

31. Soldering Method

For SMD components, I use a temperature-controlled soldering iron and appropriate soldering technique.

Before soldering a component:

- Identify pin 1 or polarity marking.

- Check the footprint orientation.

- Confirm the component value.

- Tin the appropriate pad if necessary.

- Position the component.

- Solder one pin first.

- Check alignment.

- Solder the remaining pins.

- Inspect the solder joints.

For fine-pitch ICs, I inspect carefully for solder bridges.

A microscope or magnification tool is useful for small components.

32. Post-Assembly Inspection

After assembly, I do not immediately connect the battery.

I first inspect:

- Power pins

- Ground connections

- ESP32 pins

- Sensor pins

- MOSFET pins

- Connector polarity

- Solder bridges

- Missing components

I then use a multimeter to check important power rails for unexpected shorts.

33. First Power-Up

The first power-up is performed carefully.

The procedure is:

PCB inspection

↓

Continuity / short check

↓

Power polarity check

↓

Current-limited power source

↓

Check input voltage

↓

Check regulated voltage

↓

Check ESP32

↓

Check sensors

↓

Check motor-control system

I avoid connecting the motors during the earliest electrical tests.

This makes troubleshooting easier and prevents a firmware or hardware error from immediately becoming a mechanical problem.

34. ESP32 Programming

I use Arduino IDE for the initial firmware development.

The basic process is:

- Install the appropriate ESP32 board support.

- Connect the programming USB interface.

- Select the correct ESP32 board.

- Select the correct COM port.

- Compile the firmware.

- Upload the firmware.

- Open Serial Monitor.

- Verify startup messages.

- Test each hardware subsystem.

During development, serial output can be used to display:

System startup

IMU detected

BMP280 detected

ToF sensor detected

Battery voltage

Sensor values

Motor status

System state

35. Sensor Testing

I test each sensor independently before combining the sensors into flight-control software.

IMU Test

I verify:

- Accelerometer output

- Gyroscope output

- Sensor communication

- Sensor update rate

- Axis direction

BMP280 Test

I verify:

- Temperature

- Pressure

- Estimated altitude

- Sensor communication

ToF Test

I verify:

- Distance readings

- Sensor communication

- Measurement stability

- Field of view

Testing sensors individually makes it easier to determine whether a problem is caused by hardware or software.

36. Motor Testing

Motor testing is performed only after the control electronics are verified.

The four outputs are tested separately:

M1 → Motor 1

M2 → Motor 2

M3 → Motor 3

M4 → Motor 4

I verify:

- Correct output

- Correct motor connection

- Correct motor direction

- Stable operation

- No unexpected heating

- No abnormal current consumption

Initial testing is performed without propellers installed.

37. Flight-Control Development

The flight-control software can be divided into several layers.

Sensor Reading

↓

Sensor Filtering

↓

Attitude Estimation

↓

Flight Controller

↓

Motor Mixing

↓

Motor Output

The IMU provides acceleration and gyroscope data.

The software processes the data to estimate the drone's orientation.

A control algorithm then determines how much correction is required.

The motor mixer converts those corrections into four motor commands.

38. Motor Mixing

A quadcopter requires the four motors to work together.

The flight controller changes individual motor outputs to control:

- Throttle

- Roll

- Pitch

- Yaw

For example:

Roll correction

→ Increase power on one side

→ Decrease power on the opposite side

The exact motor-mixing equations depend on the physical motor layout and rotation directions.

These parameters must be verified against the actual assembled drone.

39. Battery Monitoring

Battery monitoring can be implemented using an appropriate voltage-divider circuit connected to an ESP32 ADC input.

The basic concept is:

Battery +

|

R1

|

+------ ESP32 ADC

|

R2

|

GND

The resistor divider reduces the battery voltage to a level that the ADC can safely measure.

The resistor values must be selected so that the maximum possible battery voltage remains within the ADC input limits.

The firmware can then calculate the approximate battery voltage.

40. Debugging Method

When something does not work, I use a systematic debugging process.

I do not immediately replace components.

I first check:

- Power

- Ground

- Physical connections

- Component orientation

- Communication

- Firmware

- Signals

- Hardware behavior

For example, if an IMU is not detected:

Check power

↓

Check GND

↓

Check SDA

↓

Check SCL

↓

Check I2C address

↓

Check pull-ups

↓

Check sensor orientation

↓

Check footprint

↓

Check firmware

This approach reduces unnecessary component replacement.

41. Common Problems

ESP32 Does Not Start

Possible causes:

- Incorrect power voltage

- Short circuit

- Incorrect regulator output

- Incorrect boot configuration

- Damaged ESP32

- PCB soldering problem

Sensor Not Detected

Possible causes:

- Incorrect power

- Wrong I2C address

- SDA/SCL swapped

- Missing pull-ups

- Incorrect footprint

- Incorrect sensor orientation

- Firmware configuration problem

Motor Does Not Run

Possible causes:

- Incorrect motor connection

- MOSFET problem

- Incorrect gate signal

- Firmware problem

- Insufficient battery current

- Damaged motor

- Incorrect connector

ESP32 Resets When Motors Start

Possible causes:

- Battery voltage drop

- Power-supply instability

- Motor electrical noise

- Poor grounding

- Insufficient decoupling

- Excessive current demand

This is why motor testing and logic-system testing should be performed separately during development.

42. Tools I Use

The project can be developed using the following tools:

Hardware Tools

- Soldering iron

- Solder

- Flux

- Multimeter

- Tweezers

- Wire cutters

- Wire strippers

- Magnification / microscope

- Laboratory power supply when available

- USB cable

- Programmer / USB interface

Software Tools

- KiCad

- Arduino IDE

- ESP32 board support

- Serial Monitor

- GitHub

KiCad is used for schematic and PCB design.

Arduino IDE is used primarily for firmware development.

GitHub is used to store and document the project.

43. Parts Selection Philosophy

When choosing components, I consider more than just price.

My priorities are:

- Electrical compatibility

- Datasheet specifications

- Physical size

- Current capability

- Voltage rating

- Availability

- PCB footprint

- Reliability

- Weight

- Future expandability

For critical components, I prefer to check the manufacturer's datasheet before creating the PCB footprint.

44. Mechanical Design

The electronics are only one part of the drone.

The mechanical structure must also support:

- PCB

- Battery

- Four motors

- Propellers

- Sensors

- Wiring

- Connectors

The PCB should be mounted securely.

The IMU should be mechanically stable.

The battery should be firmly secured so that its movement does not affect the center of gravity.

The final structure should also prevent wires from interfering with the propellers.

45. Weight Considerations

Drone performance is strongly affected by total weight.

Important components include:

- PCB

- Battery

- Motors

- Propellers

- Frame

- Wires

- Connectors

- Sensors

The PCB should therefore be designed with a reasonable balance between:

strength + electrical performance + functionality + weight

Removing unnecessary copper or components is not always beneficial. Structural reliability and adequate electrical performance must come first.

46. Safety and Testing

I treat the battery and motors as the highest-risk parts of the hardware during testing.

I perform initial electrical tests without propellers installed and use current-limited laboratory power when appropriate.

I do not rely only on firmware behavior to determine whether a board is electrically safe.

Before a first flight test, I verify:

- Battery installation

- Motor direction

- Propeller orientation

- Motor mounting

- PCB mounting

- Sensor operation

- Control response

- Failsafe behavior

- Wireless connection

- Battery condition

47. Current Project Status

| Item | Status |
| --- | --- |
| Original drone investigation | Completed |
| Original PCB repair | Successfully completed |
| KiCad schematic | Developed |
| PCB layout | Developed |
| Component placement | Developed |
| PCB routing | Developed |
| DRC inspection | Performed |
| Gerber generation | Prepared |
| Drill file generation | Prepared |
| JLCPCB manufacturing preparation | In progress |
| ESP32 firmware | In development |
| Sensor integration | In development |
| Motor integration | In development |
| Full flight testing | Not completed yet |

48. What I Learned

This project has taught me more than just how to build a drone.

I learned how to:

- Read a PCB.

- Trace electrical connections.

- Use a multimeter for debugging.

- Identify electronic components.

- Understand power rails.

- Work with low-voltage digital electronics.

- Design schematics.

- Design PCBs in KiCad.

- Assign footprints.

- Route PCB traces.

- Perform DRC.

- Prepare Gerber files.

- Prepare drill files.

- Understand manufacturing requirements.

- Work with ESP32 hardware.

- Integrate sensors.

- Design around real hardware limitations.

- Debug both hardware and firmware.

The original broken drone was therefore useful as a starting point for learning how real electronics work.

49. Problems I Encountered

During development, I encountered several problems.

Some were related to the original PCB, including unclear connections and unknown components.

Others appeared during the new PCB design, including:

- Incorrect or unclear net connections

- Component footprint problems

- PCB routing difficulties

- Power-rail mistakes

- Sensor placement considerations

- Silkscreen conflicts

- DRC errors

- Manufacturing-file preparation

Instead of treating these problems as reasons to stop, I used them as part of the debugging and learning process.

50. Future Development

My future plans include:

- Manufacture the PCB.

- Assemble the components.

- Perform electrical testing.

- Finish the ESP32 firmware.

- Integrate the IMU.

- Integrate the barometer.

- Integrate the distance sensor.

- Implement motor control.

- Implement attitude estimation.

- Add battery monitoring.

- Add wireless control.

- Add telemetry.

- Improve the PCB layout.

- Reduce unnecessary weight.

- Perform controlled flight tests.

- Create future PCB revisions based on test results.

51. Project Philosophy

I want this project to be more than a single working drone.

I want the hardware to be understandable and repairable by me.

Instead of buying a flight controller and treating it as a black box, I want to understand what every major section of the board does.

The project therefore combines:

Hardware + PCB Design + Embedded Programming + Sensors + Power Electronics + Wireless Communication + Debugging

This makes the drone a practical platform for learning embedded systems.

52. Final Summary

I originally started with a crashed wooden drone that I had bought several years earlier.

I first repaired and investigated the original electronics. This taught me how to examine an unknown PCB, trace connections, identify components, measure voltages, and diagnose hardware problems.

After successfully restoring the original board, I decided to build something of my own.

I began designing an ESP32-based drone PCB in KiCad. The new board is designed around an ESP32 and provides support for motion sensing, altitude sensing, distance measurement, four-motor control, power management, communication, status LEDs, and debugging.

The project is still under development, but it has already developed from a simple broken drone into a complete custom embedded-hardware project.

My long-term goal is to continue improving the PCB, firmware, and mechanical system until I have a drone platform that I understand and designed myself from the ground up.